To enable sound card support, you need to add one or more of the following

lines to your kernel configuration file:

device snd5 at isa? port 0x330 irq 6 drq 0 vector mpuintr

device snd4 at isa? port 0x220 irq 15 drq 6 vector gusintr

device snd3 at isa? port 0x388 irq 12 drq 3 vector pasintr

device snd2 at isa? port 0x220 irq 7 drq 1 vector sbintr

device snd1 at isa? port 0x388 irq 0 drq 0 vector sbintr

Unit numbers are:

1 for Yamaha FM synth

2 for SB/SB Pro DSP

3 for PAS PCM and Midi

4 for GUS

5 for MPU-401

If you have ProAudioSpectrum, uncomment units 3, 2 and 1

If you have SoundBlaster, uncomment 2 and 1.

If you have GravisUltrasound, uncomment 4

If you have MPU-401, uncomment 5

NOTE:  The MPU-401 driver may or may not work, and is unfortunately

unverifiable since no one I know has one.  If you can test this,

please let me know!

Also:  Some systems with the OPTI chipset will require you to #define

BROKEN_BUS_CLOCK in /sys/i386/sound/pas2_card.c.  Symptoms are that

you will hear a lot of clicking and popping sounds, like a geiger counter,

coming out of the PAS even when is not playing anything.

- Jordan Hubbard (jkh@freefall.cdrom.com)